Basics

Design radiator text for invitation of orders

Design radiator

VOGEL&NOOT Design radiators

consist of circular or flat steel precision heating- and assembly-pipes with pressure-resistant welding connections (no visible weld seams). All pipe ends sealed by means of special friction welding. Available as standard models or valve

models (SEINE-V equipped with a built-in set of valves, suitable for double-pipe installations and single-pipe installations, using a one-pipe manifold; with double-pipe installations, adjustable K (supply) factor from 0.13 to 0.75; with single-pipe installations, adjustment of the radiator’s proportion from 30% to 50%; equipped with a factory-installed valve element and a protective cap. With the BAWA-VM, the BAWA-T VM,  CAVALLY-VM, FULDA-VM and the LOWA-VM, an angled connection set with a covering rosette in matching radiator colour is enclosed. Suitable

for double-pipe operation, adjustable K (supply) factor from 0.12 to 0.48). The maximum positive operating pressure is 10 bar, (for the LOWA-VM model it is 5 bar); the testing positive pressure is 13 bar. The maximum operating temperature

is 110 Grad C. Performance verification in accordance with DIN EN 442 and OENORM (Austrian standard) EN 442. Delivered with dummy and vent plugs (the valve designs are factory-sealed), and a wall mounting set matching

the radiator colour, suitable for both horizontal and vertical radiator alignment (with the ARUN-T, used as a partition, a floor fastening set is added). Double coat painted in accordance with DIN 55900, suitable for bathroom units, prime

coat with top-quality electrophoretic coating, finished with electrostatic powder coating, Option of installing an electrical heating element ;

Topcoat: standard design RAL 9016 (Traffic White) various RAL and sanitary ware colours see the Colour chart

Product families:

KASAI Height: 1022, 1262, 1502, or 1742 mm

Length: 600, 750, or 900 mm

Depth (wall clearance included): 97–109 mm

Connections: 4 x internal thread G 1/2“, and 1 x internal thread G 1/4“ (for vent plug)

FULDA Height: 758, 1220 or 1766 mm

Length: 500, 600, 750 or 900 mm

FULDA-VM Height: 1220 or 1766 mm

Length: 500, 600, or 750

Depth (wall clearance included):

for an overall length of 500 .........146–158 mm

for an overall length of 600 .........155–167 mm

for an overall length of 750 .........171–183 mm

for an overall length of 900 .........180–192 mm

Connections FULDA: 2 x internal thread G 1/2, and 1 x internal thread G 1/4 (for vent plug)

FULDA-VM: 2 x external thread G 3/4 (for valve connection set), and

2 x internal thread G 1/2, and 1 x internal thread G 1/4 (for vent plug)

LOWA-VM Height: 790, 1222 or 1510 mm

Length: 500, 600, 700, 800 mm

Depth: (wall clearance included) 75–87 mm

Connections: 2 x external thread G 3/4 (for valve connection set),

1 x internal thread G 3/8, and 1 x internal thread G 1/4 (for vent plug)

Model: Watts:

Overall height: Number:

Overall length:

Design radiator

VELINO Height: 1600 or 1800 mm

Length: 510, 822, 1030 or 1238 mm

Depth (wall clearance included):

for an overall height of 1600 ......... 169–181 mm

for an overall height of 1800 ......... 177–189 mm

Connections: 3 x internal thread G 1/2, and 3 x internal thread G 1/4 (for vent and drain plugs)

CAVALLY Height: 796, 1196 or 1756 mm

CAVALLY-VM Length: 498, 596, 742 or 887 mm

Depth (wall clearance included):

for an overall length of 498 .........119–131 mm

for an overall length of 596 .........128–140 mm

for an overall length of 742 .........146–158 mm

for an overall length of 887 .........164–176 mm

Connections CAVALLY: 2 x internal thread G 1/2, and 1 x internal thread G 1/4 (for vent plug)

CAVALLY-VM: 2 x external thread G 3/4 (for valve connection set), and

2 x internal thread G 1/2, and 1 x internal thread G 1/4 (for vent plug)

Model: Watts:

Overall height: Number:

Overall length:

BAWA Height: 796, 1196 or 1756 mm

BAWA-VM Length: 500, 600, 750 or 900 mm

Depth (wall clearance included): 97–109 mm

Connections: BAWA: 2 x internal thread G 1/2, and 1 x internal thread G 1/4 (for vent plug)

BAWA-VM: 2 x external thread G 3/4 (for valve connection set), and

2 x internal thread G 1/2, and 1 x internal thread G 1/4 (for vent plug)

BAWA-T VM Height: 1194 or 1754 mm

Length: 500, 600, 750 or 900 mm

Depth (wall clearance included): 113–125 mm

Connections: 2 x external thread G 3/4 (for valve connection set), and 4 x internal thread G 1/2

ARUN-T Height: 1194, 1754 or 2154 mm

Length: 500, 600, 750 or 900 mm

Depth (wall clearance included):

for an overall length of 500 ......... 544–556 mm

for an overall length of 600 ......... 644–656 mm

for an overall length of 750 ......... 794–806 mm

for an overall length of 900 ......... 944–956 mm

Connections: 5 x internal thread G 1/2“, and 1 x internal thread G 1/2 (blind sleeve for floor fastening)

FATALA Height: 796, 1196 or 1756 mm

FATALA Length: 500, 600 or 750 mm

LEFT HAND Depth (wall clearance included): 97–109 mm

Connections: 2 x internal thread G 1/2“, and 1 x internal thread G 1/4“ (for vent plug)

Design radiator

PTC-ELECTRIC HEATING ELEMENT Stainless steel, with thread G 1/2; self-adjusting: the temperature-dependent PTC-heating element automatically controls the water temperature in the radiator by changing the resistance.

Wattage: 300, 600, or 900 watts at 60° C; transformer 230 volts;

Design: with safety plug (IP 64), or with safety plug and switch (IP 40)

VELINO ELECTRIC HEATING ELEMENT Stainless steel, with thread G 1/2“; with built-in thermostat and temperature-limiting switch.

Wattage: 615 watts, transformer 230 volts;

Design: with safety plug (IP 54), or with safety plug and switch (IP 40)

ELECTRIC HEATING ELEMENT Stainless steel, with thread G 3/8; with built-in thermostat and temperature-limiting switch.

G 3/8 Wattage: 300 or 600 watts, transformer 230 volts;

Design: with safety plug (IP 54), or with safety plug and switch (IP 40)

SPECIAL Chrome-plated T-piece, for the installation of an electric heating element for use with the BAWA, ADAPTER CAVALLY, FULDA, MOSEL and FATALA Design radiator models (standard design)

INFRAROT CONTROL UNIT To control room temperatures when using Design radiators. Clear LCD display with simultaneous indicators of room temperature, target temperature, operation mode and BOOST symbol. Option of installing the receiver, as a substitute for the traditional wall socket.

Design: either as a package to include a PTC electric heating element set to the required heat output, or as a single component without PTC electric heating element.

Model: Watts:

Overall height: Number:

Overall length:

Basics

Design radiator

DESIGN RADIATOR ELECTRICAL DESIGN Electrical Design radiators with a built-in PTC electric heating element consist of steel-made circular precision heating- and assembly-pipes, connected by pressure resistance welding (no visible weld seams); self-adjusting – the temperature-dependent PTC- electric heating element

automatically controls the radiator‘s heating medium temperature by adjusting the electrical resistance; equipped with a wall mounting set matching the radiator colour, for either horizontal or vertical radiator alignment; double coating in accordance with DIN 55900, suitable for bathroom units, prime coat with highest-quality electrophoretic coating, finished with electrostatic powder coating.

Coating: standard design RAL 9016 (Traffic White) Various RAL and sanitary ware colours according to the colour chart

Product families:

FULDA ELECTRICAL Height: 1220 or 1766 mm

Length: 500 or 600 mm

Depth (wall clearance included):

for an overall length of 500 ............. 146–158 mm

for an overall length of 600 ............. 155–167 mm

BAWA ELECTRICAL Height: 796, 1196 or 1756 mm

Length: 500 or 600 mm

Depth (wall clearance included): 97–109 mm

FATALA ELECTRICAL/ FATALA LEFT HAND ELECTRICAL

Height: 796, 1196 or 1756 mm

Length: 500 or 600 mm

Depth (wall clearance included): 97–109 mm

Model: Watts:

Overall height: Number:

Overall length:

VOGEL&NOOT design radiators "SEWA" and "NERO”

a trendy, fashionable and yet timeless design radiator, consisting of high-quality, rounded and flat precision steel heating and collection tubes manufactured with the pressure resistance welding method (no visible weld seams). All tube ends are sealed by way of a special rubbing welding method, cased with high-quality decorative elements. Available as centred valve connection model with integrated thermostat head. Can be connected in corner positions or in relay positions. Comes with covering collar in the radiator’s relevant colour. Maximum excess operating pressure 10 bar, excess test pressure 13 bar; max. operating temperature 110°C. SEWA model: maximum excess operation pressure 5 bar, heat output measured according to DIN EN 442 and ONORM EN 442. Comes with blind plug and vent plug as well as wall fastening set in the relevant colour of the radiator, with horizontal and vertical radiator alignment options, wet room-proof 2-layer lacquering according to DIN 55900, primed with high-quality electrophoretic dip coating. Completely lacquered with electrostatic powder coating. Optional integration of electric heating elements.

Product family:

SEWA

Heights: 1471 mm and 1685 mm

Lengths: 600 mm and 700 mm

Depth (incl. wall clearance): 90 mm

Surface design: Standard, Elegance

NERO

Heights: 1019, 1419 and 1819 mm
Lengths: 530 and 630 mm
Depth (incl. wall clearance): 106 mm - 118 mm

Surface design: Standard, Elegance

Model:

Design height:

Design length:

Watt:

No. of items:

TARA

TARA is a vertical panel radiator. It has a large, even front with beautifully designed straight lateral edges. The framed front surface is galvanised on both sides. Standard model in white RAL 9016. Also available in other RAL and sanitary colours for an additional charge.

Types and sizes:
Types 11 (design depth 73 mm) and 21 (design depth 97 mm) 1800, 1950 and 2100 mm
Design lengths:  300, 450, 600, 750 mm (not actual design length)

Operating pressure: 6 bar

Test pressure: 8 bar

Connections:
Convenient connection through a standard centred connection fitting 2x G 1/2" IG (50 mm) plus 4x G1/2” IG, including blind plug and vent plug.
If the lateral (bottom) connections are used, the flow and return flow connections of the radiator are connected from below at the 1/2" IG connection fittings.
Connection fittings are not included in the radiator.

Mounting set:
6 lugs welded to the back for installation with angular rails. Comes with a safety catch. Accessory kit with screws and dowels.

Included:

-  Blind plug ½“ 
-  Vent plug ½“ 
-  Wall fastening set

- Accessory kit with screws and dowels.

Not included:

Towel holder in the models:
- Stainless steel, matt, brush-finished. Up to 2 towel holders may be mounted at freely adjustable heights. 
Retrofitting is even possible without dismantling the radiator.

The connection fittings are not included with the radiator and are available from the range of accessories.

LEVO Plan centred valve radiator

LEVO is a vertical panel radiator with arched lateral connections and framed front surface, galvanised on both sides.

Connections

Centred connection 2 x G 1/2“ IG (50 mm) plus 4 x G 1/2“ IG. Comes with blind plug and vent plug.

Convenient connection through a standard centred connection fitting (50 mm). If the lateral (bottom) connections are used, the flow and return flow connections of the radiator are connected from below at the G 1/2" IG connection fittings.

Design heights: 1800, 1950, 2100 mm

Design depths:
Type 11: 73 mm

Type 21: 97 mm

Mounting:

6 lugs welded to the back for fastening the radiator with angular rails. Comes with a safety catch according to VDI 6035, AK 1 and 2. Accessory kit with screws and dowels.

Operating pressure 6 bar

Test pressure 8 mbar

Coating according to DIN 55 900. Highly corrosion-resistant KTL primer and finish in RAL 9016 as stoved powder enamelling. Other RAL and sanitary colours as well as metallic colours available on request. Packaged ready for installation in stable cardboard with edge protection and shrink-wrapped.

Mounting:

The mounting set required for installation is included in the scope of delivery.

Model:

Design height:

Design length:

Watt:

No. of items: