REPLACEMENT  RADIATOR

Material & surface:

Made of cold-rolled sheet steel according to EN 442, stable, well-designed profiling with 40 mm profiling intervals, primer annealed at 190°C, complete with lacquer in the form of electrostatic powder coating according to DIN 55900, part 2, in RAL 9016, annealed at 210°C object temperature.

Equipment:

Equipped with hanging brackets welded to the back, a removable top cover and two closed lateral parts (for types 21 K-S, 22 K and 33 K).

Installation:

Disassembly and installation of the upper cover by way of decorative clips (in RAL 9016), performance-tested according to DIN EN 442 und permanent production monitoring according to EN-ISO 9001. Comes with cardboard assembling aid. Packaged three-fold (cardboard, edge protection, shrink film).

Connections: 4 x G ½ I.G.

Type:

Watt:

Design height:

Items:

Design length: